# TCP Three-Way Handshake, Teardown and Flags

Prepared by: Swarnachandran Kesheth

Week 2 Theory Notes  •  Network Traffic Analysis, Nmap and Windows Logging

# Overview

TCP is a connection-oriented transport protocol. It uses sequence numbers and acknowledgements to deliver data reliably.

# Three-way handshake

| Step | Packet | Purpose |
| --- | --- | --- |
| 1 | Client → Server: SYN | Request connection; advertise initial sequence number. |
| 2 | Server → Client: SYN, ACK | Accept request and acknowledge client sequence. |
| 3 | Client → Server: ACK | Acknowledge server sequence; connection established. |

# Connection teardown

| Step | Packet | Meaning |
| --- | --- | --- |
| 1 | FIN from one endpoint | This side has finished sending. |
| 2 | ACK from peer | Peer acknowledges the FIN. |
| 3 | FIN from peer | Other side finishes sending. |
| 4 | Final ACK | Close is acknowledged; TCP may wait in TIME_WAIT. |

# TCP flags

| Flag | Meaning | Analyst observation |
| --- | --- | --- |
| SYN | Synchronize sequence numbers | Repeated SYNs without replies can indicate filtering or scanning. |
| ACK | Acknowledgement | Confirms received bytes; common in established sessions. |
| FIN | Graceful close | Normal end of a session. |
| RST | Reset | Abrupt close or response from a closed port. |
| PSH | Push data to application | Often seen in interactive data transfer. |
| URG | Urgent pointer valid | Rare; investigate unexpected patterns. |

# Practical example

Opening an HTTP page typically starts with SYN → SYN/ACK → ACK before the browser sends a request. Wireshark can show these packets by filtering tcp.

# Learning summary

I can explain the key concepts above and apply them when reviewing authorized lab traffic or Windows events. Practical results must be recorded separately during Week 2 hands-on tasks.